FIRE PLAN FOR CONSTRUCTION AND SERVICE CONTRACTS

- SCOPE:

The provisions set forth below outline the responsibility for fire prevention and suppression activities and establish a suppression plan for fires within the contract area.  The contract area is delineated by map in the contract.  The provisions set forth below also specify conditions under which contract activities will be curtailed or shut down.

- RESPONSIBILITIES:

- Contractor

- Shall abide by the requirements of this Fire Plan.

- Shall take all steps necessary to prevent his/her employees, subcontractors and their employees from setting fires not required in completion of the contract, shall be responsible for preventing the escape of fires set directly or indirectly as a result of contract operations, and shall extinguish all such fires which may escape.

- Shall permit and assist in periodic testing and inspection of required fire equipment.  Contractor shall certify compliance with specific fire precautionary measures in the fire plan, before beginning operations during Fire Precautionary Period and shall update such certification when operations change.

- TOOLS AND EQUIPMENT:

The Contractor shall comply with the following requirements during the fire precautionary period, as defined by unit administering contracts:

The Fire Precautionary Period is set by the State of California which is April 1 through December 1 of any year.

A.  Fire Tools and Equipment:  Contractor shall meet minimum requirements of Section 4428 of the California Public Resources Code (C.P.R.C.).  Fire tools kept at each operating landing shall be sufficient to equip all employees in the felling, yarding, loading, chipping, and material processing operations associated with each landing. All required fire tools shall be maintained in suitable and serviceable condition for fire fighting purposes.

Trucks, tractors, skidders, pickups and other similar mobile equipment shall be equipped with and carry at all times a size 0 or larger shovel with an overall length of not less than 46 inches and a 2-1/2 pound axe or larger with an overall length of not less than 28 inches.

B.  Fire Extinguishers:  Contractor shall equip each internal combustion chipper with a fire extinguisher for oil and grease fires (4-A:60-B:C).

All Fire Extinguishers shall be mounted, readily accessible, properly maintained and fully charged.

C.  Spark Arresters and Mufflers:  Contractor shall equip each operating tractor and any other internal combustion engine with a spark arrester, except for motor vehicles equipped with a maintained muffler as defined in C.P.R.C. Section 4442 or tractors with exhaust-operated turbochargers.  Spark Arresters shall be a model tested and approved as shown in the. National Wildfire Coordinating Group Spark Arrester Guide, Volumes 1 and 2, and shall be maintained in good operating condition.  Every motor vehicle subject to registration shall at all times be equipped with an adequate exhaust system meeting the requirements of the California Vehicle Code.

D.  Power Saws:  Each power saw shall be equipped with a spark arrester approved according to C.P.R.C. Section 4442 or 4443 and shall be maintained in effective working order.  An Underwriters Laboratories (UL) approved fire extinguisher containing a minimum 14 ounces of fire retardant shall be kept with each operating power saw.  In addition, a size 0 or larger shovel with an overall length of not less than 38 inches shall be kept with each gas can but not more than 300 feet from each power saw when used off cleared landing areas.

- GENERAL

A.	State Law:  In addition to the requirements in this Fire Plan, the Contractor shall comply with all applicable laws of the State of California.  In particular, see California Public Resource Codes.

B.	Smoking:  Smoking shall not be permitted during fire season, except in a barren area or in an area cleared to mineral soil at least three feet in diameter.  The Contractor shall sign designated smoking areas. Contractor shall post signs regarding smoking and fire rules in conspicuous places for all employees to see.  Contractor's supervisory personnel shall require compliance with these rules. Under no circumstances shall smoking be permitted during fire season while employees are operating light or heavy equipment or walking or working in grass and woodlands.

C.	Storage and Parking Areas.  Equipment service areas, parking areas, and gas and oil storage areas shall be cleared of all flammable material for a radius of at least 10 feet unless otherwise specified by local administrative unit.  Small mobile or stationary internal combustion engine sites shall be cleared of flammable material for a slope distance of at least 10 feet from such engine.

F.	Reporting Fires:  As soon as feasible but no later than 15 minutes after initial discovery, Contractor shall notify 911 emergency services of any fires on Contract Area or along roads used by Contractor.  Contractor's employees shall report all fires as soon as possible.

When reporting a fire, provide the following information:

    - Your Name

    - Call back telephone number

    - Project Name

    - Location:  Legal description (Township, Range, Section); and Descriptive location (Reference point)

    - Fire Information: Including Acres, Rate of Spread and Wind Conditions.